Technical Service Bulletin Redbox Machine Support

Subject: Top Global Firmware Upgrade.

Bulletin Number: TSB09-00086

Date Issued: 8/20/2008

Description: This will show how to apply firmware upgrades to the Top Global Routers.  You will need to have a laptop, the latest firmware upgrade and the download_nvram config file which is in a notebook format.  The 2 files are available from the thumb drive image.

1.  If you don’t already have the firmware upgrade, go to www.topglobalusa.com.

2.  Select “Support Center” and then “Download.”  You will need to put the SN from the bottom of the Top Global router in the requested field and press “Go.”

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

3.  Our router is the MB5000K.  The newest release should be the only one in the list.

4.  Save the file to your desktop.

NOTE:  The latest file is version v1.1.33-1 which will fix connection issues.  This is not available online as of the writing of this TSB.

5.  While at the kiosk, unplug the network cable from the PC and plug it into your laptop.  Reconfigure your laptop to have an IP address of 172.16.0.2 and a subnet mask of 255.255.255.0.  This will allow you to connect to the router.

NOTE:  IE is disabled on the kiosk so this is the way that we will navigate to the router.  You can also hook up the router to your laptop before ever installing it and do these steps.

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

6.  Open an IE page and type 172.16.0.1 into the address bar.  You will receive a logon prompt.

7.  Username is: r3db0x-c3ll

Password is: 2r3db0x-c3ll

8.  When you login, select Firmware on the left.

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

9.  Select “Browse” and point to the file on your desktop.  Select “Open” when finished.

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

10.  Press the “Upgrade firmware” button.

11.  Select OK.

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

12.  The firmware will upgrade and reboot the router.  Do not shut off the router.  It will reset automatically.

13.  Reconnect to 172.16.0.1 and select “Factory Defaults.”  You may need to log back in.  Refer to steps 6 & 7 above.

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

14.  Select “Yes” and the router will reboot.

15.  Reconnect to 172.16.0.1 and log back in with new credentials:

Username: public

Password: public

16.  Select “Backup/Restore”.

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

17.  Select “Browse.”

18.  This will bring up the explorer window for selecting the file to upload to the router.  Navigate to the location where you saved the download_nvram config file.  Press “Open.”

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

19.  Select “Upload Configuration” and the router will reboot.

20.  Reconnect to the router-you will have to use the logon info from steps 6 & 7 above-and select “DHCP” on the left-hand side.  This will ensure if the username and password have successfully changed to our standard one.

Date Issued: 8/20/2008

Bulletin Number: TSB09-00086

21.  Ensure that the “Enable DHCP Service” is NOT checked.

22.  You may now close the IE window and reset your PC back if you’re not doing anymore upgrades.

23.  Plug the patch cable back into the kiosk and reboot the router first then the PC.